| Fluid Resistant Respirators Note: the addition of a face shield may increase fluid protection |  |  |  |  |
| --- | --- | --- | --- | --- |
| Mask image | Details | Applicable Standards | Protection levels | Other |
|  | 3M: 1860 N95 Size: medium Box 20 | NIOSH 42, CFR Part 84 | PFE >95% BFE >99% FRL: 2 (120mmHg) | ARTG: 255656 NIOSH TC-84A-0006 |
|  | 3M: 1860S N95 Size: small Box 20 | NIOSH 42, CFR Part 84 | PFE >95% BFE >99% FRL: 1 (80mmHg) | ARTG: 255656 NIOSH TC-84A-0006 |
|  | BSN: 72509-09 N95 Size: small Box 50 | NIOSH 42, CFR Part 84 | PFR >95% BFE ≥ 98% FRL: 3 (160mmHg) | ARTG: 305431 NIOSH TC-84A-3348 |
|  | BSN: 72509-10 N95 Size: medium Box 50 | NIOSH 42, CFR Part 84 AS/NZ 1716:2012 | PFE >95% FRL: 3 (160mmHg) | ARTG: 334581 NIOSH TC-84A-3348 |
|  | BYD: DE2322 N95 Size: medium Box 25 | NIOSH 42, CFR Part 84 AS 4381:2015 GB 19083-2010 ASTM F2100 | PFE >95% BFE > 99% FRL: 3 (160mmHg) | ARTG:337767 Note: 332300 superseded in June 2020 NIOSH TC-84A-9221 |
|  | Everyday Office National (Honeywell): H910 Plus series, part number #SP910 N95 Size: regular Box 50 | NIOSH 42, CFR Part 84 EN14683:2014 EN14683:2019 | PFE >95% BFE >98% FRL: IIR (120mmHg) | ARTG: 335988 NIOSH TC- 84A-8480 |
|  | AlphaProTech: 695 N95 Size: universal Box 35 | NIOSH 42, CFR Part 84 ASTM F2100 | PFE ≥ 98% BFE ≥ 98% FRL: 3 (160mmHg) | ARTG: 331773 NIOSH TC-84A-0457 |
|  | 3M: 9320A+ P2 Size: regular Case 400 | AS/NZS 1716:2012 AS/NZS 4381:2015 AS4381:2015 | PFE ≥ 94% FRL: 2 (120mmHg) | ARTG:335620 BSI: BMP 572181 -(compliance to AS/NZS 1716:2012) |
|  | O&M Halyard: 46727 N95 Size: standard Box 35 | NIOSH 42, CFR Part 84 ASTM F2100-11 | PFE ≥ 98% BFE ≥ 98% FRL: 3 (160mmHg) | ARTG: 314855 NIOSH TC-84A-7521 |

| Non-fluid resistant Respirators Note: the addition of a faceshield may increase fluid protection |  |  |  |  |
| --- | --- | --- | --- | --- |
| Mask image | Details | Applicable Standards | Protection levels | Other |
|  | 3M: 8210 P2/N95 Size: medium Box 20 | NIOSH 42, CFR Part 84 AS/NZ 1716:2012 | PFE >95% FRL: Nil | ARTG: 334581 NIOSH TC-84A-0007 SAI Global Standards Mark License: SMKH20654 |
|  | 3M: 8110S P2/N95 Size: small Box 20 | NIOSH 42, CFR Part 84 | PFE >95% FRL: Nil | ARTG: 33458 NIOSH TC-84A-0007 |
|  | Detmold: W376 P2 Size: medium Box 25 Comes with small ear saver clip | AS/NZ 1716:2012 AS 4381: 2015 ASTM F2100 EN 14683:2019 | BFE ≥ 98% FRL: Nil claimed | ARTG: 337768 |
| Ear saver buckles/clips (may improve fit of ear loop mask) |  |  |  |  |
| Roo Technologies RT-0005-02 Box 20 |  |  | VB Precision Tooling B-1963 Box 500 |  |
| Detmold Fit Clip XL W008S0001 69mm x 19mm Box 50 |  |  |  |  |